Compressed Air Hoists:

New silent air motor and an exhaust muffler are fitted. New motor is ordered from the OEM and gets fitted by the Mine. Noise level before / after: 102 dBA to 84 dBA. (300% improvement)

Silenced Compressed air water pump

Hose is fitted to the compressed air exhaust port of the pump. Exhaust is kept in the water being pumped at all times. Noise level before / after: 93 dBA to 75 dBA. (600% improvement)

Diamond Drill Silencer

A silencer installed at the compressed air discharge port of the Diamond Drills used at BRPM, to reduce the noise level emitted. Noise level before / after: 122 dBA to 101 dBA. (400% improvement)

Mechanical Loader silencer

Two silencers installed at the compressed air discharge ports of the Mechanical Loaders used at BRPM, to reduce the noise level emitted. Noise level before / after: 114 dBA to 105 dBA. (300% improvement)

“Buy Quiet” Procurement Policy

According to the Mines Health and Safety Act, 1996, Chapter 2, Section 21 it is the responsibility of the suppliers and manufacturers to ensure that any article for use at a mine must as far as reasonable practicable be without risk to health and safety. In the event of equipment not complying with specified noise level requirements, a penalty in the form of a reduction in the purchase/repair/modified price paid for the equipment will be instituted, calculated on a proportional basis, i.e. 85 dB(A) level required compared to the actual noise level.

Compressed air control valve

These valves are used underground on main compressed air lines to control flow and pressures. These valves cannot be converted but need to be replaced with a newly built sound attenuated unit. The silenced equipment complies with the DME’s Milestones. Noise level before / after: 97 dBA to 89 dBA. (300% improvement)