- INTRODUCTION

The Impala Personal Protective Equipment (PPE) Policy focuses on the following issues:-

  - Ensuring that all our employees (this includes all contractors and sub-contractors) are protected from injury in their work environment.

  - Safeguarding the occupational health of employees from harmful health related exposures in the work environment.

  - Ensuring that employees are provided with a free issue of compulsory PPE at reasonable and specified intervals.

  - Provide for specialized clothing to be purchased at subsidized rates at intervals specified.

  - That training is given in the use and care of PPE and facilities are provided (where necessary) for the safe / hygienic storage of such PPE.

  - The conservation of PPE to the extent that unnecessary wastage and abuse is avoided.

  - All used PPE will be disposed of in accordance with the waste disposal procedure described in chapter eight of the mine standards and procedures.

- SELECTION CRITERIA FOR PPE

The selection criteria for PPE are based primarily on employee choice, provided that such choice is based on the following parameters:-

  - That selection is based on popular preference, ascertained by means of a census of at least 10% (ten percent) of employees using the item of PPE concerned.

  - As far as possible ensuring that choice is determined by showing different makes of the same item of PPE as supplied on contract.

  - Ensuring beforehand that all items are in accordance with specified contract criteria.

  - Allow other PPE suppliers not currently on contract, the opportunity of submitting items requiring census, providing that such items are not in excess of contract prices and conform to standard contract specifications.

  - Those items, as far as possible, are within the contract price ranges.

  - By accepting that cost is a major deciding factor, which could influence the final decision of the Mine Health and Safety Act 3.1 appointee or Impala Platinum Limited.

  - Employees, (especially contractors and sub-contractors) may only use approved compulsory PPE.

  - Specialized items must be authorized by the manager or engineer by a signed requisition for each issue.

- RULES FOR THE ISSUE, WEAR AND CONTROL OF PPE

  - Signage

    - As far as practicable SANS or ISO listed signage will be used on Impala.

    - Posting specifications must be such that signage will be conspicuous, legible and at a position from the risk source that will allow ample warning in terms of type of PPE required, distance from source placed and action required by person entering area.

    - Applicable PPE symbolic signs and / or notices must be posted at the entry points to risk areas. Risk areas in this context refers to areas where conditions and work being done are such that there is a possibility of people sustaining injury or ill health due to :-

      - objects falling, rolling, moving, sliding

      - people losing balance, slipping, falling, colliding

      - people handling tools, equipment, material, rolling stock

      - people operating vehicles, machinery

      - working near / in / at heat, cold, liquid, gas, noise, dust, foreign bodies, stored energy

      - restricted clearance, nip points, sharp points and edges

      - unfamiliar areas or operating requirements

  - Wear

    - All employees and visitors must don all applicable PPE before passing the signs referred to in 3.1 above.

    - The applicable PPE may not be removed until the person is outside the risk area or if all the sources of risk in the area are removed. For example, if the only risk in an area is noise from a particular rockdrill and the drill is stopped, hearing protection may be removed but must be replaced before the machine is restarted.

    - Operators must warn co-workers, visitors and bystanders in the immediate vicinity and affected area to don applicable PPE before conditions in terms of risk is changed.

    - All PPE worn at Impala must conform to Impala Platinum Holdings Limited minimum specifications. Some items are brand specific or may not be the preferred item to wear by some individuals or groups of people for good reason, but, in such cases, please follow the mine standard procedure to effect change if necessary. See standards 01.01.01 and 01.01.02.

  - Issue and Replacement

    - All items of PPE, will as far as possible, be issued on an exchange basis.

    - The official responsible for the ordering of PPE will be responsible to examine the condition of the PPE and determine whether such PPE is still useable. The responsible official is not obliged to replace the PPE, because the specified re-issuing time period has elapsed. PPE should be re-issued provided the condition of PPE warrants replacement.

    - Specialized items of PPE will be handed back to the responsible official on expiry of contract or termination of services with the mine. Failure to do so will result in the employee carrying the replacement cost of such PPE.

    - Unless otherwise specified, compulsory PPE items will be issued free of charge at a frequency of 1 (one) issue per annum.

    - If any employee exceeds the stipulated number of issues: the following applies:

      - Justifiable accidental damage to item	- free re-issue

      - Excessive wear of item			- free re-issue

      - Willful damage to item			- liable for payment of full amount at personal cost

      - Loss / theft of item			- liable for payment of full amount at personal cost

  - Head Protection

    - All hard hats must have a GME HH28 approval mark stamped on it.

    - Safety representatives to wear pink hard hats only.

    - Shaft Sinkers and shaft bank workers will wear approved white “shaft sinkers” hard hat. Crew may only wear white hats. Persons giving signals to grab operator must wear easily distinguishable coloured hat other than white.

    - Development end RDO’s must wear wide brim hard hats.

  - Eye Protection (As Specified In Code)

    - Gauze (non-prescription) eye wear issue frequency: monthly

    - Plastic frame to be issued free of charge at a frequency of 1 (one) issue every 12 (twelve) months.

    - Lenses to be issued free of charge at a frequency of 1 (one) issue every 6 (six) months.

    - Prescription lenses to be issued on employee's request (for personal account).

  - Hearing Protection

    - Hearing protection will be worn in all areas where noise levels are above 85dB (A).

    - Issue frequency of Noise Clipper: One issue per five (5) years. Temporary disposable ear plugs: Daily.

    - Damaged / lost units will be replaced after two years by Impala.

  - Hand Protection

    - To be issued free of charge at a frequency of 1 (one) issue per month. Nitral gloves one pair per six weeks.

  - Reflective Gear (e.g. Reflective bibs)

    - All employees and visitors going underground are required to wear approved reflective material. Employees not making use of standard reflective overalls or reflective tape on hard hats must wear reflective bibs over their clothing before entering any area where any vehicle or rolling stock may be used, eg. haulages.

    - All persons on surface including contractors, surveyors, security personnel and emergency services, working on roads, rail networks and at accident scenes, day and night, must wear approved reflective clothing in a manner to render themselves and their co-workers clearly visible to oncoming traffic.

    - Issue frequency 2 (two) per annum. Mat. 766175163.

  - Foot Protection

    - Issue frequency of leather boots: one (1) pair per annum. Gumboots: two (2) pairs per annum.

    - All underground workers must wear gumboots. TMM operators may use leather boots due to heat whilst operating machine.

    - Either gumboots or leather boots will be supplied where these specialized items are prescribed, not both.

4	TECHNICAL SPECIFICATIONS

4.1	HEARING PROTECTION

# 4.1	Custom-Made Hearing Protection Device (Noise Clipper)

# PURPOSE

To allow all employees (including contractors) working in noisy areas the option of being fitted with their own custom-made hearing protection devices to prevent noise induced hearing loss.

    - PROCEDURE

# The Human Resources Department will instruct all new employees to go to the Noise Clipper testing centre at the Occupational Health Centre (OHC).

# During the annual medical examination at the OHC employees will be instructed to go to the Noise Clipper testing centre as part of their annual medical examination procedure.

# On arrival, their ears will be inspected to ensure that there are no abnormalities. Any abnormalities will be

dealt with immediately.

      - A briefing session will be held with the employees, whereby the noise clipper personnel will inform them of the choice they have regarding hearing protection devices

# If opting for noise clipper the relevant information of the new employee will be recorded and an impression of the employee’s ear canals will be taken by noise clipper personnel.

# The effectiveness of the noise clipper units of the employees who have already been issued will be verified in accordance with the manufacture’s specifications. If still effective the noise clipper units will be handed in at the noise clipper testing centre for service and maintenance.

# In case of any deviations in the physical condition of the employees’ ear canals a new impression of the employee’s ear canals will be taken by noise clipper personnel.

# All impressions will be forwarded to the manufacturing site where the custom-made hearing protection devices are manufactured.

# These devices will be returned to the noise clipper testing centre within three weeks. These units will be issued to employees at the respective shafts by noise clipper personnel.

# CONTROL OF ISSUES

# If the noise clipper is lost /abused the employee will have the ultimate choice to replace it or use non custom-made hearing protection devices. If the loss / abuse happens within two years of issue the employee will be liable for the cost of impression and fitment should the employee choose to replace the noise clipper. The form in Annexure B must be used for this purpose.

# PROCESS TO BE FOLLOWED IF LOST, OR ABUSED

4.1.3.1		The employee must immediately start using non custom-made hearing protection devices and report to the H.R. Officer as soon as the loss is discovered.

      - In exceptional cases a full enquiry must be held to establish the circumstances surrounding the loss of the noise clipper

      - The presiding officer must inform the employee of the choice the employee has regarding hearing protection devices

# COMPLIANCE

4.1.4.1	Shift bosses to monitor compliance on a daily basis and take appropriate action.

4.1.4.2	Safety audit team to conduct routine compliance inspections on the wearing of hearing protection devices as part of routine inspections and report to line management.

      - Miner to report compliance on wearing of hearing protection devices in the daily hazard identification book.

      - The ventilation department will do compliance testing during routine Ventilation/Occupational Hygiene inspections and report to line management.

      - Disciplinary action may be taken against any person found not wearing his/her personal hearing protection devices in a noisy area.

      - Any problems experienced with hearing protection devices must be reported to the noise clipper testing centre at the OHC.

  - Non-Custom-Made Hearing Protection Device (Ear Plugs)

# PURPOSE

To allow all employees (including contractors) working in noisy areas the option of being fitted with an approved non custom-made hearing protection devices (X-fit) to prevent noise induced hearing loss.

    - PROCEDURE

      - The Human Resources Department will instruct all new employees to go to the Noise Clipper testing centre at the OHC.

      - During the annual medical examination at the OHC employees will be instructed to go to the Noise Clipper testing centre as part of their annual medical examination procedure.

      - On arrival, their ears will be inspected to ensure that there are no abnormalities. Any abnormalities will be dealt with immediately.

      - A briefing session will be held with the employees, whereby the noise clipper personnel will inform them of the choice they have regarding hearing protection devices

      - If opting for non custom-made hearing protection the relevant information of the employee will be recorded and will be referred to the PPE store at his/her place of work.

      - PROCESS TO BE FOLLOWED IF LOST

The employee must immediately proceed to the PPE store and collect the non custom-made hearing protection devices.

    - COMPLIANCE

Same as for Noise Clipper procedure.

- SPECIALISED ITEMS OF PPE

  - Rainwear / RDO suits (oilers) will be issued free of charge at a frequency of 2 (two) issues per annum. As far as possible itemized re-issuing should be the practice (pants do not last as long as jacket).

  - Freezer suit c/w reflective strips 10mm wide - one per arm and one on front and back, all horizontal. 210 Denier coated nylon with 100gsm 10cm diamond quilting, ribbed cuff, hood, zip and elastic waist back. Intended for shaft examination or repair work in shaft, or in accordance with the Environmental Regulations for workplaces that no worker may work in any environment where the time-weighted average dry-bulb temperature, recorded over a period of four hours, is less than 6 degree Celsius, unless the employer provides means to protect such a worker against cold. Manager must approve these and other freezer suit issues, as it cannot be strictly applied to job category only. Frequency of issue: one issue per three years. See attached matrix to determine the wind-chill equivalent temperatures.

  - If the air temperature is 6.0oC or less gloves should be used by exposed employees and total body protection such as overalls are required with the exception of a windbreak garment when the velocity exceeds 2.0m/s. (The garment should include provisions for easy ventilation in order to prevent wetting of inner layers by sweat.)

  - If the air temperature is –10.0oC or less mittens, balaclavas as well as eye protection should be used by exposed employees.

  - If the (Equivalent Chill Temperature) ECT as determined by the tables in Annexure A is -7.0oC or less additional insulating dry clothing such as freezer jackets [include specs here] must be issued to employees to maintain the core temperature above 36.0oC such as:

- Employees involved in repair or examination work in a shaft.

- Employees driving vehicles without any “cab”.

- ADMINISTRATION OF PPE

  - All compulsory PPE, excluding boots, will be administered as follows:

- If an employee is not due to have an item replaced according to the code of practice, due to practical complications, it will be replaced for the mine’s account;

- a daily report from the PPE store issuer indicating the quantity and value of such replacements, will be forwarded and signed for by the Operations Accountant.

- All ex-leave and new employees will be issued with an employment or ex-leave slip by TEBA. This slip must be presented to the PPE issuer at the hospital for the issue of compulsory PPE. On request, non-compulsory items will be issued but will be for personal account.

  - All specialized PPE (occupations such as Boilermakers, RDO’s, etc) and boots will be administered as follows:

- If an employee is not yet due to have an item replaced, an approval form will be issued by the PPE store issuer to the employee who then will have to get approval from his Manager / Engineer / HOD, if they agree that the item was justifiably damaged;

- If the Manager / Engineer / HOD is of the opinion that the damage is not justifiable, the employee will have to pay for the replacement.

- If problems with the quality of certain items are experienced, it must be communicated to the Impala Standards Committee for action.

  - All bulk issues of compulsory PPE will be administered as follows:

- A list of industrial number must accompany the order form;

- A motivation as to why a bulk issue is needed, must be submitted with the order form;

- Approval from the Mine Manager must be obtained prior to such an issue being made.

  - No bulk issue of any specialized PPE will be allowed. These issues must be restricted to individuals.

Employees / bodies responsible for the issuing of PPE to mine employees, will ensure integrity of this code of practice by maintaining a record of each employee at the shaft or work area. Such records must be reconciled against stock levels at regular intervals and audited by employees designated by managers, not less than quarterly. The above provisos are binding and as such, will be complied with.

ADDENDUM A

COMPULSORY ITEMS OF PPE

The following items will be available for issuing free of charge to employees required to wear PPE. Size may be selected by the employee for comfort. All employees must be in possession of all compulsory PPE before being allowed into the shaft bank area.

The following items will be available for issuing free of charge to employees required to wear PPE. Size may be selected by the employee for comfort. All compulsory PPE must be worn before any employee is allowed into the shaft bank area

| ITEM | SPECIFICATION |
| --- | --- |
| HARD HAT M/C 766112003 P008 P007WH P405 REFLECTIVE STICKER | Make: 909CLB - Plastic cap type with v guard Colour: White or yellow (see paragraph 3.4) Other: With P008 lining, sweat band, lamp bracket at back (not side), cable clip and a yellow reflective strip, 20mm x 500mm, visible from both left and right-hand sides and back. Reflective material to EN 471 specifications. 1. Unacceptable if hard hat does not have GME HH28 stamp of approval |
| P438PI+L & P173 (Safety Reps) | 2. Pink for safety representatives only. Alternative colours are not allowed. |
| P437WH | 3. Shaft sinkers and shaft bank workers must wear approved white "shaft sinkers" hard hat. Crew may only wear white hats. Persons giving signals to grab operator must wear easily distinguishable coloured hat, i.e., NOT white. |
| P253 | 4. Wide brim hard hat for Development RDO's |
| MINING GUM BOOTS M/C 765313216 or 765313122 P170; P819 (Shosaloza without metatarsal & steel sole insert) | Make: General purpose gumboot with steel toe cap (STC) Colour: Black PVC Other: PVC, size, knee length Comment: Comply with specifications agreed to by IMPALA, suppliers and distributors which will be higher than SABS 492 type 3 specifications in terms of wear resistance. All underground workers will wear gumboots to assist in keeping feet dry and for people to use the correct traveling paths. Wear leather spats over gumboots when doing hot work / flame cutting. Boilermakers may wear leather boots. |
| M/C 765331275/81 P217; P322; P340; P534; P535 | Make: Leather boots c/w STC (One issue per annum, surface job specific or doctor’s note only) Colour: Black / brown chrome, ankle Other: Rubber sole |
| # HEARING PROTECTION # Disposable earplug, corded # P446xft | Type: Disposable Earplug Make: Uvex X-fit Other: 1. To reduce noise exposure below 85db @ 8 hour shift length 2. Noise reduction to be specified on packaging. |
| HEARING PROTECTION # Disposable Earplug, un-corded P896 | Type: Disposable Earplug Make: Uvex X-Fit Un-corded disposable earplug (for dispensers or otherwise) Other: 1. To reduce noise exposure below 85db @ 8 hour shift length 2. Noise reduction to be specified on packaging. |
| # Noise Clipper | Type: Custom made hearing protector with noise filtering Make: Noise Clipper - To reduce noise exposure to below 85Db @ 8 hour shift - Calibrated at 120 Db - Every Noise Clipper must undergo a seal test during fitment |
| # EYE PROTECTION P542 (full face) | Type: Gauze, full face gauze, hard hat mounted Make: any approved supplier Comment: Monthly. Available to all u/ground workers except for use during hotwork eg. flame cutting, grinding, etc. |
| P698 (Millenia); P183 Uvex Eco spec; P519 GAUZE MAX SPECTICAL | Type: Safety gauze spectacle |
| P382GR (Shade 3) | Type: Safety Lens – Green (for gas cutting, grinding) Make: Astrospec 9168 Must comply to SABS 1404 : 1993 Comment: Green; anti fog; anti scratch; replaceable. |
| I-care | Type: Clip-in Prescription Lenses (for workers required to wear prescription lenses and as supplied by approved supplier on Impala contract) Make: Astrospec 9168 Must comply to SABS 1404 : 1993 Comment: Prescribed clip-in lenses to fit behind standard, clear, safety eyewear |
| REFLECTIVE BIB M/C 766175165 P027 | Type: Reflective safety vest. Reflective material to EN 471 specifications. Other: Yellow fishnet with 20 x 200mm cross on front and back (Use if reflective strips on hard hat not in place) |
| CAPLAMP BELT M/C NO. P020 (Ordinary Nylon); P565 | Other: Rubber, size 1100mm x 38mm xType: Nylon belt Other: 38mm x 1067mm Official & TM3 – belt double strap P565 |
| P151 P603 P435 | Black Web Belt Shoulder Straps For P151 Black Web Belt C/W Shoulder Straps |
| HAND GLOVES M/C 76531067 P559; P105; P786R&G | Type: Hand gloves (exchange one for one, i.e. left for left and right for right – NOT a pair if only one is damaged) Other: PVC, ultragrip, open cuff, 260mm o/length, SABS 1228 P559 (PVC Red – left & Green –right) P105 (Pigskin with candy stripes) P786 (Cotton, Red – left & Green –right ) |
| P844R&G | Type: Hand gloves Make: PVC, ultagrip, 400mm, open cuff, top padded, SABS 1228 |

REGULATION 7.8.1: No person shall work, or cause or permit any other person to work, in or near any part of the workings where inadvertent slipping or overbalancing may result in his sliding or falling down any slope that has an inclination from the horizontal of 45 degrees or more, or in his falling vertically, unless he is secured by a life-line or otherwise safeguarded.

The following items will be available for issuing free of charge to employees required to wear job specific PPE:

| ITEM | SPECIFICATION |
| --- | --- |
| HARNESS, FULL BODY M/C 766 172 201 ### P680 | Description: c/w one hookup point between shoulder blades without lanyard and without belt to SABS / EN 361 1992. Standard size adjustable to medium and large. Also available in small and extra large. Colour: Top and bottom parts to be coloured differently for easy fit. Comment: Harness must be fitted with a shackle for easy connection to or intentional detachment from the anchoring chain / lanyard / fall arrestor. |
| LANYARD M/C 766 172 211 P513 | Description: Lanyard for underground and surface use - chain, 6mm grade 300 1,5m long c/w with link T-BAR TOGGLE CONNECTOR with large links every 20th link from T-bar one end and SAFETY SNAP HOOK other end to SABS 809 including SHOCK ABSORBER to SABS EN 355. Comment: 1. Rope undesirable due to once-of test limitation, splicing control and protection requirement 2. Each chain link on chain is tested and chain certified by supplier. 3. Tracibility of chain material and components better than with polymers & cloth 4. Anchor must be able to withstand 10kN (± 1000kg) in line with SABS-EN 795 : 1992 |
| ARRESTOR, FALL M/C 766 172 221 P449 (10m SALA block) P512 (6m) | Description: c/w 10m wire rope terminating in snaphook and oval screw gate carabiner on handle for attachment purposes with test certificate to DIN specification EN 360 eg. SALA block. Colour: Any conspicuous Other: To be approved via Best Practices before introduced on Impala Comment: Only to be used if possible fall exceeds 5,75m |
| RESTRAINT BELT (Incorrectly called "Safety Belt") P017 MatCat Numbers: Small: 766 172 017 Medium: 766 172 018 Large :766 172 019 | Description: Restraint belt, polyester, adjustable, small / medium / large Colour: Any conspicuous Other: To be approved via Best Practices before introduced on Impala Comment: May only to be used at stope tip after permanent grizzly has been installed. |

4.	OPTIONAL  (Issued as per paragraph referring to optional items)

| P088 P116 P262F P264 P022 P023 P024 P052NU P050 P053 P077 P257 P850 Orange c/w reflective tape; P852 Navy & orange | PVC YELLOW NUMO JACKET PVC RAIN SUIT SCOOTER TROUSER ROYAL BLUE J54 SABS 434 (surface use only) JACKET ROYAL BLUE J54 SABS 434 (surface use only) POUCHES COMBINATION POUCHES FIRST AID POUCHES NOTE BOOK SOCKS HD MINING HAZARD TAPE white & red 150M LOCKS BRASS 40MM LOCKS BRASS 50MM SOCKS COTTON MINE CUSHION FOOT FREEZER JACKET P852 Nave & orange |
| --- | --- |
| OVERALLS: RULES FOR USE AND ISSUE | WHITE OVERALLS WILL BE ISSUED AT A FREQUENCY OF THREE PER ANNUM PER QUALIFYING UNDERGROUND EMPLOYEE AT ENGAGEMENT AND THEREAFTER EX-LEAVE, COST TO IMPALA. EXCEPT WHERE OVERALLS PROVIDE PROTECTION AGAINST EXPOSURES SUCH AS HEAT OR CHEMICALS, OVERALLS ARE NOT CONSIDERED AS PPE AND THE WEARING THEREOF IS NOT COMPULSORY. |
| U/G MALES P531 | IMPALA OVERALL, WHITE UNBLEACHED 1PC D59 C/W REFLECTIVE TAPE |
| U/G FEMALES P029 IC | IMPALA OVERALL UNBLEACHED 2PC D59 C/W REFLECTIVE TAPE |
| SURFACE ENGINEERING P434 IC | JACKET & TROUSER, BLUE 2PC D59, SABS 434 |
| P434 T | TROUSER, BLUE 2PC, D59 SABS 434 |
| P434 J | JACKET, BLUE 2PC, D59 SABS434 |
| BATTERY Bay Att. P805 | OVERALL, WHITE 1PC ACID PROOF 100% COTTON GREEN COLLAR |
| SEAT PROTECTOR M/C 766170420 P 034 | Type: Rubber seat protector, complete with straps. Tube. |
| P308 | Type: Rubber seat protector c/w casing |
| KNEE PADS M/C 766170101, PVC knee protectors P846 PVC knee protectors P454MO | Type: Rubber knee pads Other: Complete with straps, buckles and sponge lining Guards knee PVC Strocam Capstone Rubber knee pad with foam rubber lining. Requisition, authorised by Manager / Engineer, required |

5.	SPECIAL PPE ISSUES

The following items will be available for issuing free of charge to certain occupations of employees; size selected by the employee for comfort:-

| ARTISAN / FOREMAN |  |
| --- | --- |
| LOCKOUT DEVICE | Type: Scissors; 6-hole; c/w ID tag Description: As per approved standard; must be on the person except when applied or off duty. No person may lockout on behalf of someone else. |

| BANKSMAN / ONSETTER / SHAFT HELPER / CHAIRLIFT OPERATOR |  |  |  |
| --- | --- | --- | --- |
| LOCKOUT DEVICE | Type: Scissors, 6-hole Description: As per approved standard |  |  |
| BOOTS P322; P340(14&15); P534; P535 MatCat. 765336115 - 121 depending on size reqd. | Type: Leather, STC Other: Black / brown chrome, ankle with steel toe caps |  |  |
| BODY P035 / P036 | Type: Rain suit Other: medium weight PVC, 76cm jacket, plus trousers (For shaft work) |  |  |
| P451RW MatCat. 765211542 | "CAGE CONTROL" marked reflective vests limited to Banksmen and Onsetters only. "CHAIRLIFT CONTROL" marked reflective vests limited to persons controlling chairlift users only. |  |  |
| HARD HAT P437WH | Make: Double crown, 909CLB - Plastic cap type with V-guard. (same as used by shaft sinkers) Colour: White. Other: With P 008 lining, sweat band, lamp bracket, cable clip and a yellow reflective strip, 20mm x 500mm, visible from both left and right-hand sides and back. Comment: 1. Unacceptable if hard hat does not have GME HH28 stamp of approval 2. Pink for safety representatives only. Alternative colours are not allowed. 3. Shaft sinkers and shaft bank workers must wear approved white double crown hard hat |  |  |
| BATTERY ATTENDANT |  |  |  |
| HAND PROTECTION M/C No. 765521030 P142 |  |  | Type: Acid / chemical proof. White with green cuff. Other: Yellow, elbow length, 600mm o/length (for battery cell replacement) SABS spec |
| BODY M/C No. 76617111 P113 |  |  | Type: Apron Other: Yellow PVC, 120cms x 70cms Yellow acid resistant overall c/w reflective strips |
| BOOTS M/C NO. 765313122 or 765313116 (mat.cat.no. depends on size) P170; P819 |  |  | Make: General purpose gumboot with steel toe cap Colour: Yellow / black PVC Other: size, knee length Comment: 1. As per approved specifications |
| BELL LOG LOADER OPERATOR |  |  |  |
| BODY PROTECTION |  |  |  |
| M/C No. 765211542 P035; P036; |  |  | Type: Rain suit Other: medium weight PVC, 76cm jacket, plus trousers |
| P153QS |  |  | Type: Kidney belt |
| BOOTS P217; P322; P340; P534; P535 M/C Nos.765336115 - 121 depending on size required |  |  | Type: Leather, STC Other: Black / brown chrome, ankle with steel toe caps |
| RESPIRATORY P166 M/C No. 766121382 |  |  | Type: Dust mask disposable Other: FF2 homologated |
| BOILERMAKER (ALL) |  |  |  |
| CUTTING/WELDING GLOVES M/C No.765510731 ### P006 |  |  | Type: Chrome leather gloves #### Other: P006 Leather 600mm length (boilermaker/aide only); long red and green |
| ### P005 |  |  | Type: Chrome leather gloves Other: 270mm Short - wrist length (helper only) |
| APRON #### P106 M/C No.766171311 |  |  | Type: Chrome leather welding apron Other: 82 x 56cm (Boilermaker/aide only) |
| SPATS #### P122PG M/C No. 766170151 |  |  | Type: Leather Other: Spats leather short c/w straps Ankle length (Boilermaker/aide only) |
| P121PG ### M/C No. 766170152 |  |  | Type: Leather Other: Spats leather long c/w straps Ankle length (Boilermaker/aide only) |
| BOOTS P217; P322; P340; P534; P535 MatCat. 765336115 - 121 depending on size |  |  | Type: Surface: Leather or gum boots, STC Type: Underground: General purpose leather boot with steel toe cap and leather spats Other: Black/ brown chrome, ankle with steel toe caps |
| ##### HELMET #### P042; PO43 Lens; P044 (shade 10) M/C No.76115560 |  |  | Type: Welding helmet flip front Other: Helmet Flip front, fibreglass (Boilermaker/Aide Only) |
| GOGGLES ### P040 or P246 |  |  | Type: Welding, flip front Other: (Boilermaker/Aide Only) |
| SPECTACLES P234 (Uvex shade 4) |  |  | Type: Cutting/Grinding/Gas Welding Other: UVEX Astrospec green 9168 (Boilermaker assistants) |
| RESPIRATORY #### P166 M/C No. 766121382 |  |  | Type: Dust mask disposable Other: FF2 homologated (grinding) |
| FREEZER WEAR SUIT P240 |  |  | Type: W/Proof; Flame Retardant; Blue; No Reflective Tape (Power / Welding Work ) |
| CARPENTERS |  |  |  |
| SHOES #### P584 (Fram); P218 (Bova) M/C No. 75371616 |  |  | Type: Leather Other: Smooth, black / brown with steel toe cap and vulcanized |
| EYE PROTECTION #### P698S&R |  |  | Type: Grinding/timber saw/ spray painting Other: Spectacles clear Millennia |
| RESPIRATORY #### P166 M/C No. 766121382 |  |  | Type: Dust mask Other: FF2 - for timber saw |
| CHANGEHOUSE SUPERVISOR AND ATTENDANTS |  |  |  |
| BODY M/C 765121467/479 depending on size P531 |  |  | Type: Overalls Other: White, unbleached, one piece D59. P531 Fieldswear / Beslin all sizes |
| BOOTS #### P170 or P819 M/C NO. 765313122 or 765313116 (mat.cat.no. depends on size) |  |  | Make: General purpose gumboot with steel toe cap Colour: Black PVC Other: Rubber, size, knee length Comment: As per approved specifications |
| GLOVES P223 |  |  | Make: General purpose household glove Colour: Yellow PVC Other: 30cm long Comment: As per approved specifications, quarterly issue. |
| COMPRESSOR / FRIDGE PLANT OPERATOR |  |  |  |
| SHOES #### P584; P218 M/C No. 75371616 |  |  | Type: Surface: Leather or gum boots, STC Other: Smooth, black / brown with steel toe cap and vulcanized sole |
| CONVEYOR BELT ATTENDANT |  |  |  |
| RESPIRATORY #### P166 M/C No. 766121382 |  |  | Type: Dust mask Other: FF2 |
| BODY P035 / P036 |  |  | Type: Rain suit Other: medium weight PVC, 76cm jacket, plus trousers (For shaft work) |
| DEVELOPMENT RDO |  |  |  |
| BODY #### P134 (RDO suit) OR P035/6 M/C No. 765211542 |  |  | Type: Rain suit Other: medium weight PVC, 76cm jacket, plus trousers |
| BOOTS P819 P170 M/C NO. 765313122 or 765313116 (mat.cat.no. depends on size) |  |  | Make: General purpose gumboot with steel toe cap Colour: Black rubber or Yellow / black PVC Other: Rubber, size, knee length Comment: 1. As per approved specifications |
| WIDE BRIM HARD HAT M/C P253WH P437WH P438PI&L P405 Reflective Sticker |  |  | Make: 909CLB - Plastic cap type with v guard Colour: White. Other: With P 008 lining, sweat band, lamp bracket, cable clip and a yellow reflective strip, 20mm x 500mm, visible from both left and right-hand sides and back. Comment: 1. Unacceptable if hard hat does not have GME HH28 stamp of approval 2. Pink for safety representatives only. Alternative colours are not allowed. 3. Shaft sinkers and shaft bank workers must wear approved white double crown hard hat 4. Wide brim hard hat for Development RDO's |
| HAND PROTECTION P844 |  |  | Type: Gloves Make: PVC, ultra grip, 400mm, open cuff, top padded, SABS 1228 |
| DIAMOND DRILL CREW LEADER AND CREW |  |  |  |
| BOOTS #### P170; P819 M/C NO. 765313122 or 765313116 (mat.cat. no. depends on size) |  |  | Make: Surface: Leather or gum boots, STC Colour: Black PVC Comment: As per approved specifications |
| GLOVES #### P005 |  |  | Type: Leather gloves Other: Short - wrist length |
| BODY #### P134 OR P035/6 M/C No. 765211542 |  |  | Type: Rain suit Other: medium weight PVC, 76cm jacket, plus trousers |
| ELECTRICIAN (ALL) |  |  |  |
| GLOVES P094 |  |  | Type: 11KV Other: Elbow length (Switching- other) |
| ### P142 M/C No. 765521030 |  |  | Type: Acid / chemical proof Other: Yellow, elbow length, 600mm o/length (for battery cell replacement) |
| FLASH PROTECTION #### P517 |  |  | Type: Suit Other: Gentex aluminized fibreglass with Q9 thermal liner |
| RESPIRATORY #### P166 M/C No. 766121382 |  |  | Type: Dust mask Other: FF2 - for grinding |
| BOOTS P217; P322; P340; P534; P535 M/C No. 765336115 |  |  | Type: Surface: Leather or gum boots, STC Type: Underground: General purpose gumboot with steel toe cap and leather spats Other: Black / brown chrome, ankle with steel toe caps |
| BODY P035/6 M/C No. 765211542 |  |  | Type: Rain suit Other: medium weight PVC, 76cm jacket, plus trousers |
| ENGINEERING DRAUGHTSMAN |  |  |  |
| SHOES #### P584; P218 M/C No. 75371616 |  |  | Type: Leather Other: Smooth, brown with steel toe cap and vulcanized sole |
| ENGINEERING HELPER - ELECTRICAL |  |  |  |
| RESPIRATORY #### P166 M/C No. 766121382 |  |  | Type: Dust mask Other: FF2 - for grinding |
| ENGINEERING HELPER - MECHANICAL |  |  |  |
| RESPIRATORY #### P166 M/C No. 766121382 |  |  | Type: Dust mask Other: FF2 - for grinding |
| FITTER (ALL) |  |  |  |
| BOOTS P217; P322; P340; P534; P535 M/C Nos.765336115 - 121 depending on size. P534 - brown; P535 – black. |  |  | Type: Surface: Leather or gum boots, STC Type: Underground: General purpose gumboot with steel toe cap and leather spats Other: Black / brown chrome, ankle with steel toe caps |
| GLOVES #### P142 |  |  | Type: Acid, chemical proof Other: Yellow, elbow length, 600mm o/length (For pumps and settlers) |
| BODY #### P113 M/C No. 76617111 |  |  | Type: Apron Other: Yellow PVC, 120cms x 70cms(For pumps and settlers) |
| GARDEN SERVICES |  |  |  |
| BOOTS P217; P322; P340; P534; P535 M/C Nos.765336115 - 121 depending on size required |  |  | Type: Leather STC Other: Black / brown chrome, ankle with steel toe caps |
| RESPIRATORY #### P166 M/C No. 766121382 |  |  | Type: Dust mask Other: FF2 |
| LAMPHOUSE |  |  |  |
| SHOES #### P584; P218 M/C No. 75371616 |  |  | Type: Leather STC Other: Smooth, black / brown with steel toe cap and vulcanized sole |
| HAND PROTECTION #### P142 M/C No. 765521030 |  |  | Type: Acid / chemical proof Other: Yellow, elbow length, 600mm o/length (for battery cell replacement) |
| BODY #### P113 M/C No. 76617111 |  |  | Type: Apron Other: Yellow PVC, 120cms x 70cms |
| LOADER OPERATOR |  |  |  |
| RESPIRATORY #### P166 M/C No. 766121382 |  |  | Type: Dust mask Other: FF2 |
| LOCO / TRANSPORT CREW |  |  |  |
| RESPIRATORY #### P166 M/C No. 766121382 |  |  | Type: Dust mask Other: FF2 |
| BODY PROTECTION #### P153 |  |  | Type: Kidney belt Other: |
| MASON |  |  |  |
| BOOTS #### P170; P819 M/C NO. 765313122 or 765313116 (mat.cat.no. depends on size) |  |  | Make: General purpose gumboot with steel toe cap Colour: Black PVC Other: Rubber, size, knee length Comment: 1. As per approved specifications |
| MINER |  |  |  |
| TOOLS P511 M/C No. 626 121 827 P604 (Pouch) |  |  | Type: Scraper rope strand cutter with pouch; Raco midget cutter hard steel 4mm, soft steel 1,5mm, 210mm long, 6mm capacity, springloaded handles, comfort vinyl grips with lock-clip |
| PAINTER & PLUMBER |  |  |  |
| HAND PROTECTION #### P142 M/C No. 765521030 |  |  | Type: Acid / chemical proof Other: Yellow, elbow length, 600mm o/length |
| RIGGER (ALL) |  |  |  |
| BOOTS P217; P322; P340; P534; P535 M/C 765336115 - 121 depending on size required |  |  | Type: Surface: Leather or gum boots, STC Type: Underground: General purpose gumboot with steel toe cap and leather spats Other: Black or brown chrome, ankle with steel toe caps |
| GLOVES #### P005 |  |  | Type: Chrome leather gloves Other: Short - wrist length |
| BODY M/C 765211542 Jacket P035 Trousers P036 |  |  | Type: Rain suit Other: medium weight PVC, 76cm jacket, plus trousers |
| ROCKDRILL / SMALL MACHINE MAINTENANCE SHOP OPERATORS |  |  |  |
| SHOES #### P584; P218 M/C No. 75371616 |  |  | Type: Leather Other: Smooth, black / brown with steel toe cap and vulcanized sole |
| GLOVES #### P142 M/C No. 765521030 |  |  | Type: Acid, chemical proof Other: Yellow, elbow length, 600mm o/length |
| BODY #### P113 M/C No. 76617111 |  |  | Type: Apron Other: Yellow PVC, 120cms x 70cms |
| RESPIRATORY #### P166 M/C No. 766121382 |  |  | Type: Dust mask Other: FF2 - for rockdrill testing chamber (rockdrill shop only) |
| SALVAGE YARD SUPERVISOR / HELPER |  |  |  |
| SHOES #### P584; P218 M/C No. 75371616 |  |  | Type: Surface: Leather or gum boots, STC Type: Underground: General purpose gumboot with steel toe cap and leather spats Other: Smooth, black / brown with steel toe cap and vulcanized sole |
| APRON #### P106 M/C No.766171311 |  |  | Type: Chrome leather welding apron 82 x 56cm Other: work related |
| HAND PROTECTION #### P142 M/C No. 765521030 |  |  | Type: Acid / chemical proof Other: Yellow, elbow length, 600mm o/length (for battery cell replacement) |
| RESPIRATORY #### P166 M/C No. 766121382 |  |  | Type: Dust mask Other: FF2 - for timber saw |
| SCRAPER WINCH OPERATOR |  |  |  |
| GLOVES #### P006 |  |  | Type: Nitral, bottom padded or chrome leather short Other: Elbow length |
| RESPIRATORY #### P166 M/C No. 766121382 |  |  | Type: Dust mask Other: FF2 |
| TOOLS P511 (Cutter); P604 (Pouch) |  |  | Type: Scraper rope strand cutter with pouch; Raco midget cutter hard steel 4mm, soft steel 1,5mm, 210mm long, 6mm capacity, springloaded handles, comfort vinyl grips with lock-clip (Pouch) |
| SECTION CONSTRUCTION CREW |  |  |  |
| CUTTING/WELDING P129 |  |  | Type: Chrome leather gloves padded Other: 600mm length (crew leader only) |
| GLOVES #### P005 M/C No.765510731 |  |  | Type: Chrome leather gloves Other: Short - wrist length, (helper only) for shotcreting - elbow length rubber gloves |
| APRON #### P106 M/C No.766171311 |  |  | Type: Chrome leather welding apron Other: (Crew leader only) |
| BODY M/C 765211542 Jacket P035 Trousers P036 |  |  | Type: Rain suit Other: medium weight PVC, 76cm jacket, plus trousers |
| RESPIRATORY #### P166 M/C No. 766121382 |  |  | Type: Dust mask Other: FF2 - for shotcreting |
| SPATS P122PG; P121PG M/C No. 766170151 M/C No. 766170152 |  |  | Type: Leather Other: Ankle length (Crew leader only) Other: Spats leather short c/w straps Ankle length (Boilermaker/aide only) Other: Spats leather long c/w straps Ankle length (Boilermaker/aide only) |
| BOOTS P217 P322 P340 P534 P535 M/C Nos.765336115 - 121 depending on size required |  |  | Type: Leather Other: Black / brown chrome, ankle with steel toe caps. Crew leader and assistant only. |
| SEWAGE / DETRITUS SUPERVISORS AND HELPERS |  |  |  |
| HAND PROTECTION #### P142 M/C No. 765521030 |  |  | Type: Acid / chemical proof Other: Yellow, elbow length, 600mm o/length (for battery cell replacement) |
| BODY M/C No. 765211542 Jacket P035 Trousers P036 |  |  | Type: Rain suit Other: medium weight PVC, 76cm jacket, plus trousers |
| BOOTS #### P170; P819 M/C NO. 765313122 or 765313116 (mat.cat.no. depends on size) |  |  | Make: General purpose gumboot with steel toe cap Colour: Black PVC Other: Rubber, size, knee length Comment: 1. As per agreed specifications 2. Subsidize as per mine rules |
| SHAFT TIP HELPER |  |  |  |
| LEG PROTECTION #### P455 |  |  | Type: Leg guard, long Other: Lower leg, rubberized |
| STOPE RDO |  |  |  |
| BODY M/C 765211542 P035; P036; P134 |  |  | Type: Rain suit Other: medium weight PVC, 76cm jacket, plus trousers. Jacket P035; Trousers P036 |
| BOOTS #### P170; P819 M/C NO. 765313122 or 765313116 (mat.cat.no. depends on size) |  |  | Make: General purpose gumboot with steel toe cap Colour: Black PVC Comment: As per approved specifications |
| HAND PROTECTION P844 |  |  | Type: Gloves, PVC red & green, reinforced 400mm Centurion |
| STORE YARDMAN / STORES ISSUER (SURFACE ONLY) |  |  |  |
| SHOES #### P584; P218 M/C No. 75371616 |  |  | Type: Leather Other: Smooth, black / brown with steel toe cap and vulcanized sole |
| RESPIRATORY #### P166 M/C No. 766121382 |  |  | Type: Dust mask Other: FF2 |
| BODY #### PT04; P393 M/C No. 766151118 |  |  | Type: Dust coat Other: Blue, bleached, cotton, with breast pocket |
| SURVEY / SAMPLING CREW |  |  |  |
| RESPIRATORY #### P166 M/C No. 766121382 |  |  | Type: Dust mask Other: FF2 (Survey peg drilling/diamond saw operator) |
| BODY M/C No. 765211542 Jacket P035 Trousers P036 |  |  | Type: Rain suit Other: medium weight PVC, 76cm jacket, plus trousers (Diamond saw operator only) |
| SWEEPING CREW |  |  |  |
| RESPIRATORY #### P166 M/C No. 766121382 |  |  | Type: Dust mask Other: FF2 |
| TM3 OPERATORS |  |  |  |
| BODY PROTECTION #### P153QS |  |  | Type: Kidney belt |
| RESPIRATORY #### P166; P386; P387; P388 ; P367, P368, P369, P371, P600 M/C No. 766121382 |  |  | Type: Dust mask. Make: Moldex half mask respirator P367; Everyguard Abek 1 cartridge P368; Everyguard P3 particulate filter – 8080 single - each P369; Everyguard particulate filter Hol – 8090 P371; Cartridge for respirator (P368) – P600 Other: FF2 #### P166 dust mask; P386 single resp (Everyguard); P387 Resp half mask double (Everyguard); P388 cartridges P2 dust for P386 &387 |
| BODY P035 / P036 |  |  | Type: Rain suit Other: medium weight PVC, 76cm jacket, plus trousers (For shaft work) |
| BOOTS P217 P322 P340 P534 P535 M/C Nos.765336115 - 121 depending on size required |  |  | Type: Leather Other: Black / brown chrome, ankle with steel toe caps. |
| WATER JETTING CREW |  |  |  |
| BODY M/C 765211542 P035; P036 |  | Type: Rain suit Other: medium weight PVC, 76cm jacket, plus trousers Jacket P035; Trousers P036 |  |

TABLE TO DETERMINE THE EQUIVALENT CHILL TEMPERATURE - ECT

TABLE TO DETERMINE THE EQUIVALENT CHILL TEMPERATURE - ECT

Annexure B

M E M O R A N D U M

___ Shaft

TO:				Noise Clipper

FROM:		Accountant					DATE:		__________________

SUBJECT:		Re- issuing of Noise Clipper

The following employee must be re-issued with a noise clipper:

| Industry Number |  |
| --- | --- |
| Full Name |  |
| Surname |  |

The reason for re-issuing is due to: (Please mark with an X)

| Damaged / Broken (damaged / broken noise clippers must be attached to this form or a copy of submission to time office) | Stolen (Relevant Statements) | Lost / Other (Please specify in the space provided below) |
| --- | --- | --- |

Reason for damage, broken, stolen, lost or other: (please attach relevant documentation)

Please note: If the damaged / broken noise clipper do not accompany the wearer of this letter or if proof is not attached, the re-issuing cost will be for the employees own account unless permission is obtained from the Mine Manager.

(Please mark with an X)

| Self Payment | The employee agrees to pay an amount of R550 for his noise clipper which will be deducted from his salary in installments of R68.75 over a eight (8) month period. Deductions only to commence after noise clipper have been fitted. |
| --- | --- |
| Company Cost | Cost Centre: |

I the undersigned hereby declare that the information supplied is correct:

Employee__________________________

I the undersigned declare that the necessary proof has been inspected and found to be in order:

Mine Overseer/ Engineer:	 ________________________________

Human Resources: 		 ________________________________

Accountant:			 ________________________________ 	Accountant’s Stamp

Mine Manager:		 ________________________________